### PIR Sensor

- GND on PIR → PIN 39 on Raspberry Pi (Ground)

- OUT on PIR → PIN 40 on Raspberry Pi (GPIO 21)

- VCC on PIR → PIN 2 on Raspberry Pi (5V Power)

### IR Raspberry Pi-Camera

### Temperature-Humidity Sensor

- GND on DHT22 → PIN 14 on Raspberry Pi (Ground)

- DATA on DHT22 → PIN 16 on Raspberry Pi (GPIO 23)

- VCC on DHT22 → PIN 4 on Raspberry Pi (5V Power)

### Light Sensor

- Red Wire on TCS34725 → PIN 1 on Raspberry Pi (3V3 Power)

- Black Wire on TCS34725 → PIN 6 on Raspberry Pi (Ground)

- Blue Wire on TCS34725 → PIN 3 on Raspberry Pi (GPIO 2 SDA)

- Yellow Wire on TCS34725 → PIN 5 on Raspberry Pi (GPIO 3 SCL)

- Orange Wire on TCS34725 → PIN 9 on Raspberry Pi (Ground)

- Green Wire on TCS34725 → PIN 20 on Raspberry Pi (Ground)

Resource: https://www.waveshare.com/w/upload/b/bb/TCS34725_Color_Sensor_user_manual_en.p